The Indirect Method of Mosaic

A How-To Outline by George Fishman

Page 1

This procedure describes a contemporary version of opus vermiculatum using

readily-available porcelain mosaic materials. (Some photos illustrate other techniques,

but the installation procedure is essentially the same.)

Draw Original Design

A. Consider design's relation to technique that you choose.

1. Opus Vermiculatum (squiggly tile paths) w/ similar-sized modules.

Slowest process. Even meandering tile pathways. See "Dove".

2. Mix of vermiculatum w/ tesselatum (grid) background. Faster process.

Juxtaposition of curvilinear foreground elements in opus vermiciculatum

against rectilinear pattern of opus tesselatum background. See "Maya".

3. Mix of sizes of modules, ie. 1/2x1/2" w/ 1x1" tiles. Faster process.

4. Shattered tile technique. Contemporary style. Fastest progress.

5. Whatever the technique, create detail in the design that's no

smaller than what's pracitcal for the size of mosaic module you want

to work with.

B. Consider site/setting.

1. Practical issues

a. Surface evenness, hardness, slickness.

2. Stylistic appropriateness to other design elements of site.

C. Choose design medium.

1. Traditional media

2. Computer drawing/painting programs

Select Tile Colors

A. Use original drawing as reference, but let tiles "talk" to each other.

1. Sometimes tile colors are prescribed by surrounding elements of site

and determine palette of design.

2. Sometimes availability of commercial tiles determines palette.

a. Some mosaic materials, ie. Byzantine glass, are avail. in many

more colors than other mosaic materials, ie. marble, stone-look

glazed tile.

b. Making your own ceramic materials gives greater breadth of

possibilities, but increases cost.

B. Unless strong graphic-look is desired, select more than one shading of each

principal color, esp. those that will cover largest areas, to create depth.

Begin Cutting Stock of Tesserae for Opus

Vermiculatum

A. Nip most of the 1x1" tiles into quarters, but cut some into halves, for custom

shaping later.

B. Sort them by color in containers.

Tutorial, Page 2

Cover Original, Full-Size Drawing W/ Clear Or Frosty

"Contact Paper", Or Equivalent

A. Allows drawing to be seen as layout proceeds.

B. Allows easy readjustment, while preventing accidental jostling.

Lay Out Tesserae On Contact Paper.

A. Establish principal contours first.

B. Use drawing as reference, but elaborate and simplify as you go.

C. Create halo of background around principal elements for one or two courses,

then veer off into tesselated or wavy background.

D. Remember that the movement of the courses of tesserae is esp. important in a

relatively simple design.

E. Adjust spacings using fingers and tweezers to avoid the need for tiny slivers to

fill areas. Often by sliding a group of tess. left or right you can close a small

"blank" space or open it enough for a whole or half tesserae.

F. If something looks "off", try substituting a group of alternative tesserae, simply

placing them on top of the ones in place and squinting to see if you have an

improvement, then plucking out and substituting.

G. Avoid nipping directly over your layout; the shower of particles and slivers

tends to coat the contact paper and make it less sticky. In Other words, don't

do what the photo shows you!

H. Keep regrouping your extra tesserae by color families. Avoid a confusing

mess and have easier time locating the next color you need.

I. When you think you're done, go away and do something else, then return to look

again and make adjustments and refinements.

Cover Mosaic With Clear Adhesive Film Or Glued

Paper

A. Cut manageable sized strip from roll.

B. Drape strip between hands and carefully place.

C. Wipe from center toward ends and edges of tape to avoid wrinkles and

bubbles.

D. Continue overlapping strips until surface is covered.

Cut Cement Backer Panel If Mosaic Is Not Installed

On Site

A. For straight cuts use a razor knife to score the panel, then snap it off against

the edge of a table. For circles or other contours, use a jig saw with a masonry

blade.

B. Save a scrap to do a test sample of the mounting procedure. Later on it will

serve as a guage to let you know when the thinset on the panel is arriving at the

proper time to peel the film off.

Drill Holes For Mounting Hardware Or Wire.

A. Use a 1/2" diameter masonry bit. Drill all the way through, but stuff a bit of

paper into the holes, so thinset won't fill them up during the mounting procedure.

B. Don't attach any wires or hardware now, because they would interfere w/ next

steps.

Mix Thinset.

A. Use latex-enhanced thinset adhesive. Refer to instructions on container.

Laticrete International's slection of sanded grouts, mixed with their #4237 liquid

additive, gives a color choice of thinset/grout combination. This method is

"self-grouting", so thinset color becomes grout color.

B. Start w/ proper quantity of latex additive, esp. important with impervious, "hard

to stick" materials like porcelain and glass.

C. Add powder, a little at a time, mixing with margin trowel to consistency of

oatmeal or grits. Be sure to scrape bottom of bucket to stir up any stray pockets

of dry powder.

D. Wait fifteen minutes for material to slake. Stir again.

Flip Mosaic 'Sandwich' And Peel Backing.

A. Fold contact paper back on itself and peel tight. Don't pull straight up, as this is

more likely to detach tesserae. If any come loose, just stick them back down.

Moisten Substrate Or Panel W/ Sponge.

Backbutter Mosaic, Using Rubber Grout Float.

A. Be attentive to angle of trowel and pressure. Hold trowel at approx. 45¼ angle

and maintain firm pressure to force thinset/grout through the joints.

B. If some tesserae become dislodged, don't panic, just pick them out w/

tweezers and toss. They can be replaced later.

Butter The Substrate, Using Notched Trowel.

A. Use smooth edge first to create a good even smear on the panel.

B. Switch to notched edge to create even pattern of notch tracks.

C. Be sure there's enough mud to fill all the teeth, but pull excess off and scrape

back into bucket.

The Next Three Steps Are For

Panel-Mounting, Not Site Installation

Lift Panel And Set Onto Buttered Mosaic.

A. Try to place accurately, but if you're a little misaligned DON'T LIFT OFF TO

TRY AGAIN. Either some of the tesserae will pull loose or gobs of thinset will

come off - or both - creating thick and thin areas and a general mess. Just

proceed with next step and make adjustment after panel is flipped.

Beat Evenly To Create Good Bond.

A. Use block & hammer or clean rubber float

B. Start at center and work out in spiral. Avoid too much pressure at edge of

panel, or it may squeeze tight at that point and open up on opposite side.

Carefully Flip Panel Onto Its Back- Finished Side

Up.

- Adjust placment if necessary by hitting glancing blows with rubber trowel.

- Beat In With Block & Hammer Or Rubber Trowel.

- A. Start at center and spiral out. Beat flat, not with edge or heel of trowel.

- B. Some mud may squish out along edges, but don't worrry about it until you've

- finished beating in.

- C. Use finger or margin trowel to clean up edges.

- Wait Several Hours.

- Test Sample By Peeling Carefully.

- A. If tesserae pull loose, stop and wait another hour. Try again.

- B. When test shows good bond, peel it and test the thinset that's squished

- through. Poke with fingernail or pencil. It should be firm, but able to be dented with

- effort.

- Peel Face Film From Mosaic.

- Brush Surface Evenly With Bristle Brush To

- Accentuate The Grout Joints.

- Wipe Away Excess With Moist, Not Wet Sponge.

- Mix And Apply Grout

- A. Mix a new batch of grout and admix to match color that was squeezed through

- joints from back during backbutter procedure.

- B. Apply grout w/ rubber grout float using firm wiping /scraping strokes.

- 1. If practical, lightly dust surface with grout powder to maintain color,

- especially if bright or dark grout color was chosen.

- C. Wait approx. 5-15 minutes, depending upon temperature/ humidity/consitency

- of grout mix.

- D. Wipe surface w/ barely moist rags until cleared of grout film.

- 1. Use minimum of water to maintain strength and color of grout.

- If Mosaic Is Mounted On A Panel, Rather Than A

- Wall Or Floor, Allow To Cure For A Few Hours

- Before Flipping Onto Face

- Attach Hanging Wire

- A. Make a knot at either end of a length of heavy braided mirror hanging wire that

- is cut to the proper length. Epoxy the knots into the two holes you drilled into back

- of panel.

- OR

- B. Clean up drilled holes w/ a bit of epoxy, so they aren't crumbly and use the

- hole(s) to catch the heads of nails pounded into the wall at proper spacing. A

- neater alternative is to glue short sections of brass or copper tubing into the

- holes.

- C. Glue a shim near the bottom of the piece to make it hang evenly on the wall, if

- you wish.

Materials

A. Porcelain mosaics (glazed and/or unglazed), commonly available in 1x1" and

2x2" sizes. Order UNMOUNTED or snip off the rubber dots that interconnect the

tiles in 12x24" sheets. Best color range available thru tile distributers. Glass and

ceramic tesserae are available from a variety of Sources.

B. 18" wide roll clear "Contact Paper" brand adhesive film or equivalent. Use as

bottom layer of mosaic "sandwich". Sold in hardware and discount stores.

C. 48" wide roll clear Low Density PolyEthylene film (LDPE). Use as top layer of

sandwich. When ordering, request that standard 48" wide roll be slit into more

convenient 8" widths. Sold by industrial tape distributers.

D. Cement backer panel for mounting mosaic. Several manufacturers in U.S. and

abroad. Available in thicknesses from 1/4" to 1/2". Sold by lumber yards and tile

distributers.

E. Thinset, grout, latex admix. Available through lumber yards and tile distributers.

Tools

A. Drawing/design instruments & supplies.

B. Tile nippers, tweezers, razor knife.

C. Various trowels, grout float, rags, buckets.

- (Optional) jig saw.

****

From:             MJ43@aol.com

Date sent:        Sun, 9 Feb 1997 21:07:39 -0500 (EST)

To:               athena@newworld.bridge.net

Subject:          Fwd: CONCH Mosaic METHOD (fishman/aol)

---------------------

Forwarded message:

From:	athena@bridge.net (M.-J.)

To:	MJ43@aol.com

Date: 97-02-01 18:23:29 EST

Subj:  Plastic film transfer

Date:  95-12-28 14:49:31 EST

From:  GeoFISH111

You can use this same clear plastic film method as Glasimage provided on

12/21  to transfer a layout of tile or glass onto another surface -- a

wall, for instance.

Actually, I use two layers of plastic. First I do my layout (finished side

up) on the clear Contact paper  which holds everything in place as I work

on it. Then when I'm satisfied with the layout I cover the finished side

with a layer of a stronger-adhesive (low density polyethylene LDPE) clear

film. This makes a sandwich strong enough to allow transport of the

assembly to wherever it's going to be installed. I then flip the

"sandwich", carefully peel off the Contact paper layer, apply thinset both

to the mosaic surface and to the wall. (Applied as skim coat to mosaic with

a hard rubber grout trowel; applied to wall with a notched steel trowel --

1/8 to 1/4 notch, depending upon the thickness of your mosaic material.)

Finally I lift the "buttered" mosaic by the corners, place it against the

wall and beat in with a hard rubber grout trowel. After a few hours the

clear film can be peeled and the mosaic surface brushed to emphasize the

grout joints. The backbuttering step makes this a "self- grouting" method,

but if there are voids, a normal grouting procedure will finish the job.

Contact paper is avail. everywhere. For LDPE film  look up Tapes-Indust. in

Yellow Pages where you are, or order from Tape Connection Inc. in Miami, FL

(305) 863-8882. Sold in 4-ft. rolls that they will cut down to more

convenient 8-12 inch widths.