Mosaic from AOL and bungi …

We carry the mosaic cutter.  It retails for $21.00.

E-mail or call me at 1-800-966-6667.

Jenna

Meredith Stained Glass

Subj:  Re:Answer for preformed stones

Date:  96-08-24 17:45:54 EST

From:  Glassimage

An outfit called "PAC" is marketing a bonding cement for putting glass

mosaices on preformed stones.  The technique is to cut your pattern using

Pb pattern shears.  Past the pattern pieces on your glass with the front of

the pattern down and cut the glass.  Grind if there are any pieces that

should be round (such as a sun) and aren't. Place a piece of clear contact

paper over the pattern with the sticky side up.  Place the glass pieces on

the contact paper with the right side toward the paper.  Coat the wet stone

with the PAC bonding agent (enough for one 14" square stone should retail

for about $2) using a notched trowel.  Place the glass down on the bonding

and the bonding dries, remove the plastic.  PAC also sells a colored grout

(~$4 for one 14"stone). and they sell 24 patterns (~$15 each) which you can

just peel off and stick to the glass.  However, with this technique, it is

necessary to build a dam around the stone before you grout it.  For any

shape except rectangles this can be a hard job.  That's why PAC only has

square patterns.  I think a person could cut a strip of vinyl for floors

and wrap around a round stone to make a dam.

****

I saw some people asking about the cement roller and I dug out a catalog

from my recycling--

it's called Improvements, and they sell a rolling cement mixer that can be

used for cement or mortar.  It says it mixes up to 60 lbs of concrete and

water at once. It costs $39.99.  Their phone # is 1-800-642-2112.

****

Subj:  Re:Cement or concrete?

Date:  96-12-09 12:11:16 EST

From:  CUngv

I can't answer your question about the difference, if any, between concrete

and cement, but I can tell you a little about grout.  I have made four

masoic stepping stones using the method described by Vicky Payne in her

mosaic kit.  You don't need the kit to make them.  Buy  concrete stepping

stones in whatever size or shape you prefer.  I used 12 inch round ones,

about $1.50  at Home Depot.

Choose a design and copy it (carbon paper works) onto the stone.  Cut

out your design pieces from glass leaving at least 1/8  inch in between

pieces for the grout.   Glue on the pieces after they are all cut.  I use a

glass laminating glue, or the white glue that Vicky Payne sells.  For a

nice background I use little squares of a single color glass glued in

straght lines.  Let the glue over night.  Home Depot also sells several

colored,  or plain white sanded grout.  The sanded is recommended for glass

mosaic.   Add water to the grout mix, and fill in the spaces in your

project.  Let dry 20-30 minutes, then wipe away the excess with a barely

moist sponge.  Wait another 30 minutes, then wipe clean your glass. Polish,

if desired.  Wait a day or two before placing it outdoors.

I haven't tried the mold method because the pre- made stones are so

convenient and the whole process seems quicker.  I would be interested in

hearing from someone who has tried both methods who can discuss the pros

and cons.    Carolyn.

*****

I just got a sale flyer from Delphi and they have the 41/4 inch

tile/coaster form for $3.95 I think.  I have been looking for these for a

while as I want to make tiles for a bathroom remodel.

*****

http://www.rscnet.com:80/glass2u/stained-glass.html

*****

Subj:  Re:Smalti Italian mosaic tiles

Date:  95-10-02 23:53:14 EST

From:  BobK987021

Here is a recipe for cementing mosaic tile to a cementous surface(concrete

block with a brown coat of plaster):

Ingredients:  White cement

Gray (or black) cement

Silica sand

Latex bonding agent (laticrete Latex Thinset Mortar

Additive #4237)

Recipe:  10 parts white cement

10 parts gray cement

10 parts silica sand

10 parts latex bonding agent

Mix into a clean 5 gallon bucket with about 1½ - 2 inches water

Stir with mixer in drill until the consistency of cake icing

Remix occassionally to keep from hardening

****

Subj:  Re:Smalti Italian mosaic tiles

Date:  95-10-07 22:04:00 EST

From:  BobK987021

Here is a grout recipe:

Grouting:

1.  Mask off all edges of mosaic with duct tape and plastic, particularly

ground.

2.  Mix grout.

Ingredients:	White cement

Gray cement

Recipe:	Light/medium gray color:

1 part white cement

1 part gray cement

Mix into 5 gallon container with plenty of water to a 					consistency

of cake batter (runny).

Medium/dark gray color:

1 part white cement

2 part gray cement

Mix into 5 gallon container with plenty of water to a 					consistency

of cake batter (runny).

3.  Apply to mosaic, brushing on with stiff bristle.  Work grout into all

joints.

4.  Clean with water and sponge, rinsing frequently.

****

Subj:  Re:adhesives & grout

Date:  95-10-19 23:51:40 EST

From:  BobK987021

I don't know much about premixed tile adhesives, but I've got a friend in

the tile business.  He recommends thinset with a latex polymer additive

plus something called frostproof to retard moisture penetration.  You could

also try epoxy (like Deco-Rez 3513 -this is a commercial product, about $70

for two part gallon containers; or PC-7 - this is a hardware store item).

I have used the Deco-Rez on outdoor autonomous panels that have NOT taken

direct rain/snow contact.  They have endured extreme temperatures (-15 to

-40 F) without any failures.  For grout, you can either use commercial

grout products, or -- if you time it right -- aggregate material like what

is used on asphalt shingles.  The aggregate is sprinkled on while the epoxy

is soft and yields an attractive dalle-de-verre matrix appearence.

****

Subj:  Directions fpr Stepping Stones

Date:  95-12-21 20:17:49 EST

From:  Glassimage

Stepping Stones

Cut glass to match a pattern that fits inside your mold. Leave at least 1/4

inch between the pattern and  the edges.  Lay the glass on a copy of the

pattern  (it can be glued on using stick glue) smooth (right) side up.

Cover the glass with a piece of clear contact paper with the sticky side

down on the glass.  Turn the glass/plastic sheet over and adjust the glass

if necessary.  Press all pieces firmly on the plastic.  Place the glass,

plastic side down into a mold that has been treated with a release material

(Vasoline, Pam, etc. )    Mix enough "topping" mix or brick mortar mix to

cover the glass with a 1/4 inch mixture and pour over glass.  Make sure it

fills into cracks.  Tap the bottom of the mold to get out air bubbles.  Mix

the remainder of the topping mix with the premix concrete and water to a

dry consistency.  Fill the form half full.  Lay reinforcing wire such as

rabbit wire or fence wire into the mold.  Fill the remainder of concrete

and level.  Let dry 5 days.  Empty the stepping stone out of the mold.

Remove the plastic.  Clean concrete off of the glass with a scrubber.

Application of a weak hydrochloric acid solution will clean off any

concrete that is stubborn.   About 30 days is required for the stone to

reach full strength.

****

Subj:  Re:Italian micro-mosiacs

Date:  96-04-28 13:34:35 EST

From:  Mpat 2353

sven warner @ MountainTop Mosaics in Vermont-I am reluctant it took me over

1 year of intensive work..calling Italy...faxing...research...and I am a

fairly sucessful mosaic artist in colorado spgs

best of luck to all. Sven is a very polite,professional man.

****

Subj:  New Round Mosaic Form

Date:  96-05-15 17:20:20 EST

From:  StndGlass1

For those of you who have been caught up in the frenzy of stained glass

mosaic stepping stones, there are now 14" Round molds available!!   We

carry them for $18.95 if anyone is interested.  You can use them a million

times!

Jenna

Meredith Stained Glass

1-800-966-6667

****

Subj:  Re:Needed:  Mosaic Votive He

Date:  96-06-01 21:20:14 EST

From:  BASHFLRNBO

Try using a hammer to actually break the glass between 2 pieces of

newspaper (safety glasses, please).  We use a product called Elmer's Squeez

n' Caulk to glue our glass to a 3 dimensional object.  When tightly glued

and dry (approximately 24 hrs), we apply a latex reinforced grout (found in

tile departments of hardware stores).  We clean excess wet grout off of

glass by first using a squeegee and then followed by a nearly dry cheese

cloth  Wait approx. 1 hr and clean haze off glass using a dry cheese cloth.

*****

Subj:  Re:Italian micro-mosiacs

Date:  96-06-08 01:10:22 EST

From:  CraftEJP

I too, have been trying to find these tiles (practically impossible); but I

just received by glass order from Glass Crafters Stained Glass Supply, and

they included a flyer announcing that they are now carrying them. They are

vitreous tesserae imported from Italy; they are sold by the sq. ft. in

three price groups ranging from $10.50 to $24.50 a sq. ft.; even 24k gold

for $24.00 a dozen. You can reach them at 1-800-422-4552 .  I hope I can

get in an order before word gets out!  Hope this helps...Elizabeth

CraftEJP@AOL.com

****

Subj:  Re:Mosaic Lantern

Date:  96-07-02 22:33:34 EST

From:  D Glswrks

Just received a copy of "Lanterns" by Carolyn Kyle and Chuck Berets.  It

has a lot of very nice lantern patterns but one in particular is a mosaic

style that would look great outdoors with the

stepping stones everyone has been making.  The instructions are really

complete and easy to follow.

I've been making some stones with a main floral design then filling in

around it with scraps using a mosaic cutter.  The mosaic cutter is like a

scissors and nips pieces of glass off in rectangular or irregular pieces.

Perfect for using up scraps and it's really quick and effective.

Both book and mosaic cutter are available at your local glass shop or you

can E-mail me for

info.

****

Subj:  Re:Mosaic-

Date:  96-07-08 14:50:15 EST

From:  Glassimage

One of my favorites is to glue a piece of mirror (it can be any size)

usxing silicone to a piece of plain glass that is slighlty larger than the

mirror.  Then glue pieces of glass all around to decorate it.  Then grout

for a nifty project.  By the way, no need to buy expensive grout.  A brand

called TEC (sold by Lowes and others) will give a tough, plastic finish.

It can be colored with the acrilic paints sold in craft shops or any other

water soluble paint.

****

There was a lot of mosaic news at the Nashville show.  Diamond Tech now has

a line of stepping stone molds to compete with the "Tiffiany Line".  There

were 10 shapes, with the most unusual being the 18" moon shapped and the

tree ring form (6 of them makes a 2' diam. circle.  They looked a little

shallower than the current one but at the present time they are not

scheduled to be cheaper than the existing one.  Cutter Productions (Vicki

Paynes) had a line of molds also.  It appeared that they would wholesale

somewhat cheaper than the others.

****

Subj:  Re:Mosaic

Date:  96-08-13 12:14:18 EST

From:  Helene5945

Diamond Tech developed a special Mosaic Stone for the  tile and coaster

form which will not crack as regular cement does.  2 lbs tubs will be

available from your local dealer.  Look for DTI's new forms and shapes to

be coming soon.  DTI will make current stepping stone sizes and new sizes

which include the 4 1/4" square you were asking about (used for tiles and

coasters), 4" x 8" x 3" brick, stone and door stop form, 16" square, 11

5/16" x 11 5/16" x 16" triangle, 8" diameter circular form, 18" moonstone

form, 6" house shaped border stone form, 2' diameter tree and garden ring

(6 make a circle).  Beautiful patterns which were so popular at the show

will also be available.  I read that someone mistakingly thought these

forms were thinner than current forms, they are not but they are made with

high impact styrene plastic with straighter sides and improved larger lip

designed to take stones out of molds with ease.  These new fun shapes were

the hit of the AGSA show and I can't wait to see what beautiful, creative

projects people will make (one of the benefits of working at DTI is when

artisits send in their finsihed works).  Many people have called asking for

the forms and they are coming very soon.

****

of the old European churches. Sven Warner of Mountaintop Mosaics keeps a

stock of 60 luscious colors; they're  irregular shapes, approx.

3/8"x1/2"x5/8" and sold by the kilo. Not cheap but unique. For many designs

they're small enough that you don't have to nip them.

Sven also carries the Bisazza line of venetian glass

800 564-4980.

****

If you have large areas of grout (say 1/2 inch) you need to cover the

grout with a damp cloth for two days to make sure it doesn't crack on drying.

****

sea shells (have to seal them with acrylic before you grout) and broken

dishes.

****

From:             MJ43@aol.com

Date sent:        Sun, 9 Feb 1997 21:12:52 -0500 (EST)

To:               athena@newworld.bridge.net

Subject:          Fwd: tips

---------------------

Forwarded message:

Subj:    tips

Date:    97-02-04 01:40:36 EST

From:    Caressa1

To:      MJ43

HI ALL,

I HAVE A SUGGESTION FOR THOSE OF YOU WHO THINK YOU HAVE OLD WORN OUT DIAMOND

GRINDING BITS....

CALL ANY INDUSTRIAL DIAMOND MANUFACTURER (I USE NATIONAL DIAMOND LAB IN L.A.,

CALIF) AND ORDER A FEW "SOAP STONES".  RUN YOUR GRINDER AND PRESS THE STONE

INTO THE DIAMOND.  IT WILL CLEAN ANY CLOGGED AREAS & MAKE YOUR DIAMOND BIT

SEEM LIKE NEW.

**

To get a dark patina on your vase cap do the following,  steel wool the vase

cap, add salt to your black patina, rub on with a brush.  Looks great and is

a lot easier!

**

http://www.artglassworld.com/wwwboard/index.html.

88

Date:  97-02-02 20:37:42 EST

From:  Concraft

Can't believe I just came across this board.  I do mosaics and have for

years.  I made my first table 23 years ago.  I am taping a segment on an

indoor decorators birdbath on Lifetime TV's "Handmade by Design" to be shown

in late spring or early summer.  I also sold a similar design to Better Homes

& Gardens "nature & Floral Crafts" special edition magazine to come out in

the spring or summer issue. I work with mixed media which makes it very

interesting.  I have a working and teaching studio and teach mosaic framed

mirrors which fills up every time I offer it which has been once a month for

the last 6 months.  I'd be happy to share. You can email me at Concraft.

Adios amigos!

I just made a stepping stone and it turned out fine.  On top of the glass

I put down about a 1/4" layer of brick mortar.  The rest of the mold

(which was an 8x8" cake pan) was filled with concrete.  You can get both

of these from places like Home Depot.  They come in 40-60 lb bags for

about $2-3.  Pretty cheap.